# Overview

This document contains the necessary information to build the Horizontal Paintbrush Stand.

Figure 1: Horizontal Paintbrush Stands. Variant A on the left, Variant B on the right. Show mounted with a magic arm, a C-Clamp, and a C-Clamp Adapter.

Contents

Overview	1

Maker Checklist	4

Maker To Do List	4

Items to Give to User	4

Customization Guide	5

3D Printing Guide	6

3D Printing Summary	6

3D Printing Settings – Variant A	6

3D Printing Settings – Variant B	6

Post-Processing	6

Assembly Guide – Variant A	7

Required Components	7

Required Personal Protective Equipment (PPE)	7

Assembly Instructions – Variant A	8

Step A.1	8

Step A.2	9

Assembly Guide – Variant B	10

Required Components	10

Required Personal Protective Equipment (PPE)	10

Assembly Instructions – Variant B	11

Step B.1	11

Step B.2	11

Step B.3	12

Step B.4	13

Quality Check	14

Mounting Options	15

Magic Arm + C-Clamp	15

Materials Required	15

Instructions	15

Loc-Line + C-Clamp	16

Materials Required	16

Instructions	17

# Maker Checklist

This list provides an overview of the steps required to build and deliver the device to a user.

## Maker To Do List

- Read through the Maker Guide to become familiar with required components, tools, supplies, and safety gear and overall assembly steps.

- Talk to User about customization options

  - Variant A or B

  - Color

  - Number of brush slots

  - Brush slot spacing

  - Mounting Options

- Update Design if necessary

- Provide requestor with estimated cost of material and shipping.

- Order hardware components

- 3D Print components

- Gather tools, supplies, and safety equipment.

- Assemble the device

- Quality check the device

- Print “User Guide”

## Items to Give to User

- Assembled, tested brush stand

- “User Guide”

# Customization Guide

# 3D Printing Guide

## 3D Printing Summary

| Metrics | Variant A | Variant B |
| --- | --- | --- |
| Total Print Time (min) | 3h14m | 9h45m |
| Total Number of Components | 2 | 12 |
| Typical Total Mass (g) | 42 | 85 |
| Typical Number of Print Setups | 1 | 1 |

## 3D Printing Settings – Variant A

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support(Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| _A_Base.stl | 1 | 3h12m | 42 | 20 | N | 0.2/0.4 |  |
| _A_Cap.st | 1 | 0h3m | 1 | 20 | N | 0.2/0.4 |  |

## 3D Printing Settings – Variant B

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support(Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| _B_Base1.stl | 2 | 4h25m | 39 | 20 | N | 0.2/0.4 |  |
| _B_Base2.stl | 2 | 4h25m | 39 | 20 | N | 0.2/0.4 |  |
| _B_Nut_Insert.stl | 1 | 0h35m | 6 | 20 | N | 0.2/0.4 |  |
| _B_Cap.stl | 1 | 0h9m | 2 | 20 | N | 0.2/0.4 |  |
| _B_Pin.stl | 6 | 0h9m | 2 | 20 | N | 0.2/0.4 | Better to print separate |

## Post-Processing

- Remove any burrs. May be painted if desired.

# Assembly Guide – Variant A

## Required Components

BOM - 3D Printed Base A - 3D Printed Cap A - ¼”-20 UNC Tee Nut Insert, 5/16” long

## Required Personal Protective Equipment (PPE)

- n/a

## Assembly Instructions – Variant A

### Step A.1

Insert the Tee Nut into the top of the Base. Use a ¼”-20 Washer and bolt to tighten and secure the Tee Nut into the Base. Nut Holder.

### Step A.2

Add the Cap to cover the Tee Nut.

# Assembly Guide – Variant B

## Required Components

BOM - 2X Base B.1 - 2X Base B.2 - 1X Nut Insert - 1X Cap B - 6X Pins - 1X Tee Nut - Superglue

## Required Personal Protective Equipment (PPE)

- Eye protection (Superglue)

- Protective Gloves (Superglue)

## Assembly Instructions – Variant B

### Step B.1

Using 4 pins for alignment, use superglue to attach the sides of the bases together.

### Step B.2

Insert and glue the 2 remaining pins with the nut insert to secure the two halves.

### Step B.3

Insert the Tee Nut from the top. Use a ¼”-20 Washer and bolt to tighten and secure the Tee Nut into the Nut Holder.

### Step B.4

Add the Cap to cover the Tee Nut.

# Quality Check

- Ensure the Tee Nut is installed correctly.

- Check to make sure there are no sharp edges on the stand.

# Mounting Options

There Horizontal Brush Stand has a ¼”-20 UNC female thread on the bottom, so it is compatible with a variety of commercial and DIY mounting options.

## Magic Arm + C-Clamp

The Magic Arm / Articulating Arm provides a convenient way to adjust the position and orientation of the stand and can be locked in place with a single knob. Using the C-Clamp and C-Clamp Adapter is a cost-effect way of providing a sturdy, non-permanent connection to a table or other work surface. The size of the C-clamp can be chosen to accommodate different table thicknesses.

### Materials Required

Magic Arm

- https://www.amazon.ca/Koolehaoda-Adjustable-Articulating-Compatible-Microphones/dp/B08PC21P2D

C-Clamp

- https://www.homedepot.ca/product/bessey-2-1-2-inch-drop-forged-c-clamp-with-1-3-8-inch-throat-depth/1000816966

C-Clamp Adapter

- https://makersmakingchange.com/project/c-clamp-threaded-adapter/

### Instructions

- Remove any unneeded hardware from the Magic Arm until just the male thread is left on both side.

- Connect one side of the Magic Arm to the Paintbrush Stand. A locking washer or thread locker can be used to form a more reliable connection.

- Connect the other end of the Magic Arm to the C-Clamp Adapter. A locking washer or thread locker can be used to form a more reliable connection.

- Position the C-Clamp Adapter in an appropriate place on the work surface and use the C-clamp to clamp it in place. To provide better grip or protect the surface, a thin piece of rubber or DYCEM non-slip mat can be used.

## Loc-Line + C-Clamp

### Materials Required

1/2” Loc-Line Modular Hose (Enough segments to achieve desired length)

- https://www.bulkreefsupply.com/loc-line-modular-hose-segments.html (CAN)

- https://www.amazon.ca/Loc-Line-Coolant-Component-Copolymer-Segment/dp/B006R9ORIY (CAN)

- https://www.modularhose.com/ (USA)

1/2” Loc-Line Double Socket (SKU 90614)

- https://www.bulkreefsupply.com/loc-line-double-socket.html, SKU 202755 (CAN)

- https://www.modularhose.com/Loc-Line-90614 (USA)

1/2” Loc-Line Flat Surface/Fixed Mount

- https://www.amazon.ca/Loc-Line-Coolant-Component-Acetal-Copolymer/dp/B006T5JOV6 (CAN)

- https://www.modularhose.com/Loc-Line-51895?quantity=1&custcol_item_colors=8, SKU 51895-Blk  (USA)

½” Loc-Line Modular Hose Assembly Pliers (Optional)

- https://www.modularhose.com/Loc-Line-60533?quantity=1&custcol_item_colors=8 (USA)1/2” Loc-Line Hose Assembly Pliers (Optional)

- https://www.amazon.ca/Loc-Line-78004-Hose-Assembly-Pliers/dp/B0025PUF1M (CAN)

¼”-20 UNC SHCS x

C-Clamp

- https://www.homedepot.ca/product/bessey-2-1-2-inch-drop-forged-c-clamp-with-1-3-8-inch-throat-depth/1000816966

C-Clamp Camera Mount Adapter

- https://makersmakingchange.com/project/c-clamp-threaded-adapter/

### Instructions

- Attach the ½” Loc-Line Flat Surface/Fixed Mount to the bottom of the Horizontal Brush Stand using a ¼”-20 UNC screw and a ¼” lock washer.

- Attach the ½” Loc-Line Double Socket to the ½” Loc-Line Flat Surface / Fixed Mount.

- Attach the other ½” Loc-Line Flat Surface/Fixed Mount to a C-Clamp Camera Mount Adapter using a ¼”-20 UNC screw and a ¼” lock washer.

- Determine how many ½”